DOSING PUMP INSTRUCTION MANUAL

Model: MCD-3-M

Power: AC 100-240V 50/60Hz

Accuracy: better than ± 3%

Size: 220x140x60mm

MCD-3-M dosing pump control box contains three dosing pump heads, the flow of each pump head, can be set depend on need, the specified range. (Each pump head Flow range 1-4999ml per day, can be divided into 24 times per day)

The MCD-3-M can working with 1 extension dosing pump MCD-3-S (sold separately) total can control 6 dosing power heads.

Thank you for use MarineColor product, please read carefully before use.

MCD-3-M Packing List

| No. | Name | Amount | Notes |
| --- | --- | --- | --- |
| 1 | Dosing pump control box | 1PCS | 3 pump heads with 3 connectors |
| 2 | Power | 1PCS |  |
| 3 | Instructions | 1PCS |  |
| 4 | Silicone tube | 3M |  |
| 5 | the 10ml measuring cylinder | 1pcs |  |

Installation

- Dosing pump control box should first be placed in a stable place and not placed in a hot humid environment.

- Connected silicone tube on inlet and outlet of the pump head.

- Connected to the power supply to complete the installation.

Basic operate

ENTER key :select

ESC  key :return

UP DOWN LEFT RIGHT key :curser locate

On the main screen display the date and time, you can use the RIGHT key to check the pump status. Press RIGHT key again, you can check the pump next start time.

Use the LEFT key return the main screen.

1-Clock Settings:

When power on, the screen display default date and time.

working status(on/off)

mm-dd-yyyy OFF

week 00:00:00

current time

- ENTER key enter the main menu.

- Use the LEFT and RIGHT key select the “SET TIME” menu.

- Press ENTER, set the current time

- Press ESC key return the main menu

2-Calibrate Pump:

Because different liquid, has different flow rate, it is better calibrate the pump before use it, use the 10ml measuring cylinder, and test the time-span of the liquid to fill 10ml measuring cylinder, then we can get the accuracy flow rate.

- ENTER key enter the main menu.

- Use the LEFT and RIGHT key to select the “PUMP CALIBRATION” menu. Then press ENTER

PUMP 1

13:50S/10ML

Use LEFT and RIGHT key to change the pump ID

- Use the LEFT and RIGHT key to change the pump ID.

- Use the UP key to make the pump working let the liquid full of the soft tube, then press DOWN key clear the value.

- Hold down the UP key, let the liquid come into the measuring cylinder, when it arrive the 10ml, and release the UP ley, it is finish the calibration.

- Change the pump ID, to calibrate other pumps.

- Press ESC key return the main menu.

3-Setup Program:

- ENTER key enter the main menu.

- Use the LEFT and RIGHT key select the “SETUP PROGRAM” menu.

Then press ENTER.

P1          CYCL/DAY  00

ML/DAY  0000.00

pump id			pump working times per day

the amount (ml) per day

- Use the LEFT and RIGHT key to select the field you want to change and use the UP DOWN key to set the value you want, after finish setup, press ESC key return the main menu.

4-Work Mode Setup:

This pump have 2 kinds of working mode:

1) Auto Mode

2)Manual Mode

On: Switch the working mode to AUTO

SWITCH AUTO OFF

off: Switch the working mode to Manual

Press the ENTER key to change it

Press ESC key to return the main menu

5-Manual Mode:

- ENTER key enter the main menu.

- Press ENTER key to enter the manual mode.

- Use the LEFT and RIGHT key to select the pump ID.

- Use the UP key make the pump working.

- Press ESC key to return the main menu.

6-Pump Working time:

- Press ENTER key to enter the SET WORKING TIME menu.

- Use the UP and down key to select the pump ID..

- Use the LEFT RIGHT UP and down key to select and set the pump start time and stop time.

- Press ESC key to return the main menu.

Expansion Unit Configuration:

If use MCD-3-S as extended pump, just use the cable come with the MCD-3-S to connect it with the MCD-3-M, then you can setup the pump4, pump5, pump6.